# Software Documentation

### 1. Software Overview

- The software runs the full Wanderer system — it handles the OLED rhythm game on the console, sends button inputs wirelessly to the stage, and drives the servo motors that move the doll's limbs.

- Platform used: MicroPython on both ESP32 microcontrollers.

- There are three main scripts: the OLED game loop (console ESP32), the ESP-NOW sender (console ESP32), and the ESP-NOW receiver + servo controller (stage ESP32).

### 2. System Logic

- User presses a directional button (up, down, left, or right) on the console.

- The console ESP32 reads the button state and checks if it is different from the last sent command.

- If it is new, the console ESP32 sends the command (e.g. "UP") wirelessly to the stage ESP32 via ESP-NOW.

- At the same time, the OLED game registers the button press — if a matching arrow note is near the hit line, the score and combo increase; if not, an error is counted.

- The stage ESP32 receives the command and tells the PCA9685 motor driver to move the matching servo.

- The servo moves the doll's limb to its active angle, pauses for 0.3 seconds, then returns to the neutral position.

- The OLED updates its animation frame (e.g. arm raised, leg out) to reflect the direction that was pressed.

- If no button is pressed for 1 second, the OLED animation returns to the idle state.